Design and Implementation of a Single-Cycle RISC-V Processor

| Sherif ElBanna Dept. of Computer Science Nile University Giza, Egypt S.Wael2404@nu.edu.eg | Obai Ashraf Dept. of Computer Science Nile University Giza, Egypt O.Ashraf2481@nu.edu.eg | Yousef Waleed Dept. of Computer Science Nile University Giza, Egypt Y.Waleed2497@nu.edu.eg | Yousef Elnady Dept. of Computer Science Nile University Giza, Egypt Y.Mohammed2444@nu.edu.eg |
| --- | --- | --- | --- |

Abstract—This paper presents the design and implementation of a fully functional single-cycle RISC-V 64-bit processor using Verilog HDL. The processor supports a subset of the RV64I instruction set, including R-type arithmetic and logical operations, I-type immediate arithmetic, memory access instructions (LD and SD), and branch instructions (BEQ and BLT). All modules—Program Counter, Instruction Memory, Register File, Immediate Generator, ALU, Control Unit, Data Memory, and PC Update logic—are individually designed and integrated into a complete single-cycle Datapath. Simulation results obtained using Vivado XSim confirm correct execution of all target instructions and produce the expected register and memory values. The design achieved 11/11 test assertions passing with zero failures.

Keywords—RISC-V, single-cycle processor, Verilog HDL, Datapath, control unit, simulation, RV64I

I.  Introduction

The RISC-V instruction set architecture (ISA) is an open-standard, royalty-free ISA that has gained significant traction in both academia and industry. Its simplicity and modularity make it an ideal candidate for educational implementation of processor microarchitectures.

This project implements a 64-bit single-cycle RISC-V processor (RV64I subset) in Verilog HDL as part of the CSC311 – Introduction to Computer Architecture course at Nile University, Spring 2026. The processor executes each instruction in a single clock cycle by connecting all functional units through combinational logic, following the classical single cycle Datapath described in Patterson and Hennessy.

The design covers the full instruction lifecycle: fetch, decode, execute, memory access, and write-back—all within one clock period. The main contribution of this work is a complete, verified, simulation-tested implementation that correctly executes a representative test program and produces all expected results.

II. DATAPATH Architecture

The single cycle Datapath is illustrated conceptually below. All stages are active simultaneously for every instruction; the multiplexers select the correct data paths based on the control signals generated by the merged Control Unit.

┌────────┐ ┌──────────────┐ ┌──────────────┐ │ PC │──▶│ Instruction │──▶│ Control │ └────────┘ │ Memory │ │ Unit │ ▲ └──────────────┘ └──────┬───────┘ │ │ Decode │ Ctrl Signals │ ▼ ▼ next_pc ┌──────────────┐ ┌──────────────┐ ▲ │ Register │──▶│ ALU │──▶ ALU Result │ │ File │ └──────┬───────┘ ┌────┴─────┐ └──────────────┘ │ │ PC Update│◀── Branch? + Imm ▼ │ (+4 or │ ┌──────────────┐ │ branch) │ │ Data │◀── write data └──────────┘ │ Memory │──▶ read data └──────────────┘ │ ┌──────▼───────┐ │ Write-Back │──▶ Register File │ MUX │ └──────────────┘

Fig. 1. Single-cycle RISC-V Datapath block diagram.

III. DESIGN Methodology

The processor was designed in a bottom-up fashion: each module was first independently implemented and verified, then integrated into the top-level CPU module. The following subsections describe each major design decision.

A. Module Partitioning

The design is partitioned into eight Verilog modules: Program Counter (PC), Instruction Memory, Control Unit (merged main + ALU control), Immediate Generator, Register File, ALU, Data Memory, and PC Update. The top-level module (RISC_V_CPU) instantiates all eight and connects them with internal wires.

B. ALU Control Merging

Per the project specification, the ALU Control Unit is merged directly into the Main Control Unit. The control module decodes the 7-bit opcode alongside funct3 and funct7 to produce the 4-bit ALU Control signal directly, eliminating the need for a separate ALU control block. This simplifies timing analysis and reduces module count.

C. Signed vs. Unsigned Comparisons

A critical design decision concerns the BLT (branch-if-less-than) instruction. RISC-V BLT performs a signed comparison. The ALU Less output is therefore computed using Verilog's $signed () cast: assign Less = ($signed(A) < $signed(B)). Without this, negative immediate values stored as two's complement 64-bit words would be treated as large unsigned numbers, causing incorrect branch behavior.

D. Memory Addressing

Both instruction and data memories use word-aligned addressing. Instruction Memory indexes by pc [11:2] (byte address divided by 4). Data Memory indexes by address [12:3] (byte address divided by 8) for 64-bit doubleword access, supporting the LD and SD instructions required by the RV64I specification.

IV. CONTROL Unit Truth Table

Table I presents the control signals generated by the merged Control Unit for each supported instruction type. The ALU Control column encodes the operation passed to ALU.

TABLE I.  Control Unit Truth Table

| Instruction Type | Opcode | RegWrite | ALUSrc | MemRead | MemWrite | MemToReg | Branch | ALUCtrl |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R-type (add/sub/and/or/xor/sll/srl) | 0110011 | 1 | 0 | 0 | 0 | 0 | 0 | f7+f3 |
| I-type Arith. (addi/andi/ori/xori) | 0010011 | 1 | 1 | 0 | 0 | 0 | 0 | f3 |
| Load (ld) | 0000011 | 1 | 1 | 1 | 0 | 1 | 0 | 0000 |
| Store (sd) | 0100011 | 0 | 1 | 0 | 1 | 0 | 0 | 0000 |
| Branch (beq/blt) | 1100011 | 0 | 0 | 0 | 0 | 0 | 1 | f3 |

V.  Module Descriptions

A. Program Counter (program_counter.v)

The Program Counter is a 64-bit register that stores the address of the current instruction. It updates synchronously on the positive edge of the clock. When the reset signal is asserted, the PC is cleared to zero. Otherwise, it loads the value of next_pc supplied by the PC Update module.

B. Instruction Memory (Instruction_Memory.v)

Instruction Memory is a 1024-element array of 32-bit words (4 KB). It is pre-loaded during simulation initialisation with the 15-instruction test program in machine code. Reading is asynchronous; the output instruction is combinationally selected by pc[11:2], providing word-aligned access. The remaining entries are filled with NOP (addi x0, x0, 0 = 0x00000013).

C. Control Unit (Control.v)

The merged Control Unit decodes opcode, funct3, and funct7 to produce seven control signals (RegWrite, ALUSrc, MemRead, MemWrite, MemtoReg, Branch) plus the 4-bit ALUControl. R-type instructions are further decoded using the concatenation {funct7, funct3}. Default values are assigned at the start of the always block to prevent latches.

D. Register File (RegisterFile.v)

The Register File contains 32 general-purpose 64-bit registers. It supports two asynchronous read ports and one synchronous write port. Register x0 is hardwired to zero; write operations targeting x0 are silently discarded by an if(rd != 0) guard on the write port.

E. Immediate Generator (ImmediateGenerator.v)

The Immediate Generator sign-extends the immediate field for I-type, S-type, and B-type instructions to 64 bits. For I-type and Load instructions, bits [31:20] are sign-extended. For S-type (Store), bits [31:25] and [11:7] are concatenated and sign-extended. For B-type (Branch), the non-contiguous bits are re-assembled into a 13-bit signed offset (with implicit bit 0 = 0) and sign-extended.

F. ALU (ALU.v)

The ALU performs eight operations selected by the 4-bit ALUControl signal: ADD (0000), SUB (0001), AND (0010), OR (0011), XOR (0100), SLL (0101), SRL (0110), and SLT (0111). Two status outputs are produced: Zero (Result == 0) and Less ($signed(A) < $signed(B)), the latter being essential for correct BLT branch evaluation.

G. Data Memory (Data_Memory.v)

Data Memory is a 1024-element array of 64-bit doublewords. Reads are asynchronous (controlled by MemRead); writes occur synchronously on the positive clock edge (controlled by MemWrite). The address is interpreted as a byte address, divided by 8 (bits [12:3]) to index the 64-bit word array, matching the RISC-V LD/SD semantics.

H. PC Update Module (pc_update.v)

The PC Update module computes the next program counter value. If Branch is deasserted, next_pc = PC + 4. If Branch is asserted, the module evaluates the branch condition using funct3, zero, and less flags: BEQ takes the branch if zero=1; BLT takes the branch if less=1 (signed). When taken, next_pc = PC + sign-extended immediate.

VI.  Test Program and Machine Code

The processor was verified using a 15-instruction test program that exercises arithmetic, logic, shift, memory, and branch instructions. Table II shows the assembly listing with corresponding machine code.

TABLE II.  Test Program Machine Code

| Word | Machine Code (hex) | Format | Assembly & Description |
| --- | --- | --- | --- |
| [0] | 0x00700013 | R-type / I-type | addi x0, x0, 7 — x0 stays 0 |
| [1] | 0x00500793 | I-type | addi x15, x0, 5 — x15 = 5 |
| [2] | 0xFFF00813 | I-type | addi x16, x0, -1 — x16 = -1 |
| [3] | 0x410788B3 | R-type | sub x17, x15, x16 — x17 = 6 |
| [4] | 0x00F8F933 | R-type | and x18, x17, x15 — x18 = 4 |
| [5] | 0x00F919B3 | R-type | sll x19, x18, x15 — x19 = 128 |
| [6] | 0x41398A33 | R-type | sub x20, x19, x19 — x20 = 0 |
| [7] | 0x05000B13 | I-type | addi x22, x0, 80 — x22 = 80 |
| [8] | 0x013B3023 | S-type | sd x19, 0(x22) — Mem[80]=128 |
| [9] | 0x000B3B83 | I-type (Load) | ld x23, 0(x22) — x23 = 128 |
| [10] | 0x00F84663 | B-type | blt x16, x15, +12 — TAKEN (branch to [13]) |
| [11] | 0x00700A93 | I-type | addi x21, x0, 7 — SKIPPED |
| [12] | 0x00000463 | B-type | beq x0, x0, +8 — SKIPPED |
| [13] | 0x06300A93 | I-type | addi x21, x0, 99 — x21 = 99 |
| [14] | 0x001A8A93 | I-type | addi x21, x21, 1 — x21 = 100 |

Branch offset calculation: The BLT instruction at word [10] (byte address 40) targets word [13] (byte address 52). The signed offset is 52 − 40 = 12 bytes, encoded as a B-type immediate. The BEQ at word [12] targets word [14] (offset = 8), but this instruction is never reached because the BLT branch is taken.

VII.  Simulation Results

Simulation was performed using Vivado XSim 2025.2 on a behavioral (RTL) simulation targeting the xc7k70tfbv676-1 FPGA part (used for its tool support; no synthesis was performed). The testbench applied a 10 ns clock (100 MHz), asserted reset for 20 ns, then ran the program for 20 clock cycles.

A. Execution Trace

The following table reproduces the instruction-level trace printed by the testbench. The PC column shows the byte address at the clock edge; note the jump from PC=40 to PC=52, confirming the BLT branch was taken.

TABLE III.  Instruction Execution Trace (Vivado XSim)

| PC | Instruction (hex) | Opcode | rd | ALU Result | Zero | RegWrite |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 0x00500793 | 0010011 | x15 | 5 | 0 | 1 |
| 8 | 0xFFF00813 | 0010011 | x16 | -1 | 0 | 1 |
| 12 | 0x410788B3 | 0110011 | x17 | 6 | 0 | 1 |
| 16 | 0x00F8F933 | 0110011 | x18 | 4 | 0 | 1 |
| 20 | 0x00F919B3 | 0110011 | x19 | 128 | 0 | 1 |
| 24 | 0x41398A33 | 0110011 | x20 | 0 | 1 | 1 |
| 28 | 0x05000B13 | 0010011 | x22 | 80 | 0 | 1 |
| 32 | 0x013B3023 | 0100011 | — | 80 | 0 | 0 |
| 36 | 0x000B3B83 | 0000011 | x23 | 80 | 0 | 1 |
| 40 | 0x00F84663 | 1100011 | — | 1 | 0 | 0 |
| 52 | 0x06300A93 | 0010011 | x21 | 99 | 0 | 1 |
| 56 | 0x001A8A93 | 0010011 | x21 | 100 | 0 | 1 |

B. Final Register File Contents

After program completion, the register file contained the values listed in Table IV, as printed by the testbench.

TABLE IV.  Final Register File Values

| Register | Decimal | Hex | Source Instruction |
| --- | --- | --- | --- |
| x0 | 0 | 0x0000000000000000 | Hardwired zero |
| x15 | 5 | 0x0000000000000005 | addi x15, x0, 5 |
| x16 | -1 | 0xFFFFFFFFFFFFFFFF | addi x16, x0, -1 |
| x17 | 6 | 0x0000000000000006 | sub x17, x15, x16 |
| x18 | 4 | 0x0000000000000004 | and x18, x17, x15 |
| x19 | 128 | 0x0000000000000080 | sll x19, x18, x15 |
| x20 | 0 | 0x0000000000000000 | sub x20, x19, x19 |
| x21 | 100 | 0x0000000000000064 | addi x21, x0, 99 then +1 |
| x22 | 80 | 0x0000000000000050 | addi x22, x0, 80 |
| x23 | 128 | 0x0000000000000080 | ld x23, 0(x22) |

C. Data Memory Verification

The SD instruction wrote 128 to byte address 80, stored at data memory index 10 (= 80 ÷ 8). The testbench confirmed: Mem[80] = 128. The subsequent LD instruction read this value back correctly into x23 = 128.

D. Branch Verification

The BLT instruction at PC=40 compared x16 (= −1, signed) with x15 (= 5). The ALU produced Less = 1 using signed comparison ($signed), causing the PC to jump from 40 to 52 (offset = +12), correctly skipping the two intervening instructions. The final value x21 = 100 confirms the branch target (L1: addi x21, x0, 99) followed by (END: addi x21, x21, 1) were executed correctly.

E. Summary of Assertions

TABLE V.  Testbench Assertion Summary

| Expected | Source | Result | Actual Value |
| --- | --- | --- | --- |
| x0 = 0 | Hardwired zero | PASS | x0 = 0 |
| x15 = 5 | addi x15, x0, 5 | PASS | x15 = 5 |
| x16 = −1 | addi x16, x0, −1 | PASS | x16 = 0xFFFF…FFFF |
| x17 = 6 | sub x17, x15, x16 | PASS | x17 = 6 |
| x18 = 4 | and x18, x17, x15 | PASS | x18 = 4 |
| x19 = 128 | sll x19, x18, x15 | PASS | x19 = 128 |
| x20 = 0 | sub x20, x19, x19 | PASS | x20 = 0 |
| x22 = 80 | addi x22, x0, 80 | PASS | x22 = 80 |
| x23 = 128 | ld x23, 0(x22) | PASS | x23 = 128 |
| x21 = 100 | branch L1 + addi 99 + addi 1 | PASS | x21 = 100 |
| Mem[80]=128 | sd x19, 0(x22) | PASS | Mem[10]=128 |

All 11 assertions passed with 0 failures, confirming full functional correctness of the single-cycle processor for the given test program.

VIII.  Conclusion

This paper presented the complete design and Verilog implementation of a single-cycle RISC-V 64-bit processor. All eight functional modules were implemented, individually verified, and integrated into a working datapath. The merged Control Unit correctly generates all control signals for R-type, I-type, Load, Store, BEQ, and BLT instructions.

A critical design issue—the need for signed comparison in the ALU Less output for BLT—was identified and resolved using Verilog's $signed() cast. The final simulation, run under Vivado XSim 2025.2, produced 11 passing assertions and 0 failures, confirming correct execution of the full 15-instruction test program and meeting all expected results (x19=128, x20=0, x21=100, x23=128, Mem[80]=128).

Future work could extend this design to a pipelined implementation with hazard detection and forwarding, or add support for the full RV64I instruction set including JALR and U-type instructions.

Acknowledgment

The authors thank the Nile University ITCS School and the CSC311 teaching staff for their guidance throughout this project.

References

[1] D. A. Patterson and J. L. Hennessy, Computer Organization and Design RISC-V Edition: The Hardware Software Interface, 2nd ed. Morgan Kaufmann, 2020.

[2] A. Waterman and K. Asanovic, "The RISC-V Instruction Set Manual, Volume I: Unprivileged ISA," RISC-V International, 2019.

[3] Xilinx/AMD, "Vivado Design Suite User Guide: Simulation," UG900, 2025.

[4] S. Palnitkar, Verilog HDL: A Guide to Digital Design and Synthesis, 2nd ed. Prentice Hall, 2003.